# Computer Vision

## Pinhole Camera Model

The pinhole camera is an abstract model which ignores lens refraction.   Points in the focal plane are collinear with the ray originating at a point in space and passing through the optical center.

Figure 8 - Pinhole Camera

From similar triangles the mapping from points in space  to points  on the focal plane is

The projection function

simplifies  to

### Canonical Model in 3D

Points   on the focal plane at distance  from the optical center  of a camera are defined by its normal vector n and center c according to

The ray emanating from the 3D point  , passing through the lens center at the origin    and terminating at the focal plane is

where d is the optical path length.  Using  in  to solve for d, we find the intersection to be

In the canonical model, c=00f, and n=001, such that the solution in  corresponds to the similar triangles of the pinhole camera model

Suppose now our camera is rotated and translated such that the new parameters are

The equation of the plane and ray in local coordinates are as before,

with similar focal plane solution

By substituting the relations in , we may write the local solution in terms of the original coordinate frame as

The animal in  begs simplifying notation and to that end we define a system of coordinate frame bookkeeping.  Let  define a point observed in the lth reference frame expressed in terms of the kth reference frame.  The simplification is often observed.  In view of the above notation  may be expressed in terms of the canonical model as a function of only local variables

where

and

are the corresponding rigid body transformations for expressing coordinates in the local frame in the world or global frame.

In the ray equation  it is impossible to locate P because the distance along the line is unknown.  Therefore if range or position estimation are desired additional information is required.  Measuring the local position of P at an additional aperture gives rise to triangulation. Computation from such measurements requires a common reference frame which in turn requires the specification of the spatial relationship

between the cameras.  Suppose we have a second frame in which

Then the points  intersecting the planes 1 and 2 wrt to the local frames are

Transforming to a common reference frame via  we get two 3D equations

in 5 unknowns .  The solution to  is found by equating 1.32 which yields

Example 1: optical axes parallel in the standard model with C1=0,C2=00B,Rk = I,p1=(x1,0,f),p2=(x2,0,f), we obtain the standard formulas

Example 2:  canonical model + Yaw beta

For small beta

From which we see the error is proportional to fbeta.

Suppose we want  ?  We can match through the reference frame by finding equivalent coordinates of P.

Yaw FOV reduction

Invoking small angle approximations we have

Or as a fraction of the two camera FOV

FOV change for Z-shift of baseline:

Intersection of two lines in XZ plane:

Roll FOV change

Which means the reduction in vertical FOV is close to  .

Roll/rotation by

Triangulation for parallel sensor planes:  Point in space P = (X,Y,Z) pierces ref camera plane (RCF) at p=(x,y,f) and travels through camera center at C=000.  The sensor plane is

Where n=001 is the normal direction and c is the intersection of Plane1 with the z axis and is give by

c=(00f).

If only a single camera is corrected, there will be a baseline translation that is not accounted for and which results in neglibible error and FOV reduction.

### Field of View

Figure 9

Single camera FOV:

Dual camera FOV

Single camera FOV:

Dual camera FOV

Two lines

Vergence point:

Close field of view:

Up to

We have

After Zclose, field of view is equal to FOV of camera 2.

Ratio of dual to single camera fov

Two lines

Ratio of dual to single camera fov

Same cameras based on geometry:

Ratio of FOV

### Scale Ambiguity

Since x = fX/Z = kfX/kZ, two cases cannot be distinguished.  x can be either a small object close to the camera (small k), or a large object far away (large k).

## Rigid Body Transformations

## Quaternions

For our purposes, quaternions are simply a convenient booking keeping device for expressing and calculating on rotating bodies.

A quaternion  is defined as

where  satisfy the axioms

and the pure quaternion part is a vector .

The notation is odd because a quaternion is designated as a scalar, but includes a vector part.  The solution to this inconsistency is to treat quaternions as their own space.

From  the following relationships can be derived

for example,

The above relationships define a right handed cross-product coordinate set where multiplication is not commutative.

The product of two quaternions  and  is defined element wise using  such that their product  is

For brevity and when the context is clear we often omit the quaternion multiplication symbol.  In matrix form this is

or, reversing the order of multiplication,

Note that commutation does not hold.  Additionally, recognizing the dot and cross-products we can rewrite  in terms of scalar plus vector notation as

Thus  has “real part”  and “imaginary” or vector part.

The conjugate quaternion is

The dot product is defined as

The conjugate product is

where the norm is defined as

If we define the identity quaternion as . Then the inverse quaternion  is

Identities:

Rotations

Let  be a unit quaternion and  where the imaginary part is a real 3D position vector.   The product

for whatever reason turns out to represent the rotation of r defined by the quaternion p.  Using the multiplication formula twice and invoking the vector identity  we can write  as

Thus the rotated vector is

Using the identity in reverse we also have

Thus we see that the sandwiched operation cancels out the real part and we are left with a pure quaternion or equivalently another real vector in 3D space, thus we can write

One thing which remains is to determine what rotation the unit quaternion represents.  Firstly, substituting  in  yields so that we know is parallel to the axis of rotation.  Secondly, the magnitude of is unchanged.  To prove this let  where a is along q and n is perpendicular to q.  Then we have

The length preservation also implies that dot products are preserved.  So far this has all the hallmarks of a rotation operator (or reflection).  Since the triple product is preserved it must represent a rotation as the sign would be reversed under a reflection.  We can also see that compositions have the form of rotation as well, e.g. for  and , we have

Thus any two unit quaternions can be related by their product

Since we know it’s a rotation, we need to find the traversed angle.  With the orthogonal decomposition in  we want the angle between n and n’

or combining,

Adding  we obtain  or

If we subtract, we obtain instead  , or  in otherwords

where u is unit axis vector  .  Thus the unit quaternion which rotates about u by an angle theta/2 is

The redundancy of quaternions is two (because –q has the same output as q) compared with 6 for rotation matrices.

Using the angle form  in  we can find the rotated vector in terms of the angle and axis as

Matrix representation

We can write  in matrix form as

with

And substituting from the relationships above, we arrive at the angle-axis rotation matrix representation,

Component forms of  are often necessary in applications, therefore we present it here:

where we used the fact that since ,  .

In order to propagate a quaternion representation of a camera pose we must solve the differential equation governing its temporal evolution.  First we know from  that the product of quaternions (all assumed to be unity norm in what follows) represents successive rotations, therefore it should be possible to propagate a quaternion in time. Given an object with an angular speed  about axis  wrt a (fixed) world frame which we write as the pseudovector  and a similarly world referenced quaternion  after a time t+dt we expect resultant quaternion to be representable as (dropping the tildes for simplicity)

where  represents the extra rotation occurring over the interval.  The differential rotation angle about u is  and we assume the axis is constant over the interval.  Then the transition quaternion is

With some handwaving where we ignore the complex nature of quaternions (real functions are not analytic) we express the quaternion derivative as

Subsitituting the expressions  and  in  we find

Representing the multiplication as the product of the pure quaternion  the derivative becomes

If the angular velocity  is wrt the body frame, then  and substituting into() the derivative is expressed as

Can also be derived as

For  a first order solution to  is simply via

Followed by normalization because it won’t be unity anymore after the update.To solve the general differential equation  we express it in terms of the matrices as

Where  is the quaternion multiplication equivalent matrix for the pure quaternion  given in ,

For constant angular velocity over the interval  the solution is obtained by direct integration to yield

Due to its skew symmetric nature, , which can be used in a power series to resolve the matrix exponential as

The discrete time transition equation for  is then

This solution is exact for constant angular velocity over the interval and is norm preserving on q.  For situations with non constant angular rate, the integration terms in the matrix

Are accurate to second order in T since we assume a constant.  E.g. if , we are assuming zero acceleration, and the accuracy will be to second order in T.???

## Rotation Matrix Angle Axis Representation

A rotation in 3D space can be described by a unit axis vector  and a scalar angle .  The rotation is in the plane perpendicular to the axis by an amount.  The basic steps for rotating a vector  are: define the axis, define the plane, project x into the plane, rotate the result by, and finally add the result to the projection of x onto the axis to complete the rotation.

First decompose x into parts along u and perpendicular to u

where  ,, and

Define the second vector describing the plane as

The direction of the rotated projected unit vector is simply the standard formula for planar rotation of

Since rotation preserves magnitude the rotated projected vector is.  The final value for the rotated vector is the sum of the along u and rotated projected components

Using the fact that, hence  and have the same magnitude, we can write the rotated vector as

The form of the corresponding rotation matrix is known as Rodrigues’ formula

Since it is often useful to have an explicit rather than symbolic representation we expand out  record it here

Given a rotation matrix  specified in terms of Euler matrices, since the axis vector u is unchanged under R, we can compute the axis u from the eigenvector corresponding to the unit eigenvalue, e.g. .  Since both R and Rinv have no effect on u, e.g. both  and

For elements of R  and with  we can write  as

Which has solution

Using the constraint  we can solve for u3 which gives

Therefore we can find u in terms of the elements of R

The magnitude of the rotation can be found from the trace of :

From which we find

Propagation of Euler matrices

We can represent this change in angle in terms of a differential or infinestesimal rotation matrix which we now show.  Expanding the angle in  in a first order taylor series

And using sum trigonometric formulas and using small angle approximations for sin and cos yields

Where the skew-symmetric matrix

Represents the infinitesimal pitch rate.  Similarly for roll and yaw/heading, we have

The matricesare indeed rotation matrices to first order (e.g. in the limit of dt going to zero),

And they commute,

These commuting infinitesimal rotation matrices can be used to differentially propagate a general rotation matrix  by a small amount as

Which we write as

Where

Is the negative crossproduct matrix denoted  for the rate vector

The time derivative of the coordinate transformation matrix is defined as

Substituting  in  yields the differential equation for pose

When we wish to emphasize the dependence of  on the individual rates we write .

Assuming constant coefficient the solution to  is found using the matrix exponential in

Using

The closed form solution to () is

The discretized pose propagation equation over sampling interval  for the rate vector  is

where we have substituted sincs to equalize powers. For systems with small  and to first order  can be approximated by

Which for a single angle rotation, say  , is equivalent to the linear update given by .

## Stereo Cameras

Camera calibration is the process of recovering the function which maps points in space to pixels.  For multiple cameras we are interested in mapping pixels to a common coordinate frame.  This pose alignment transformation is applied to captured images in a process known as rectification.   Corresponding regions are then matched across the rectified data to facilitate higher level image understanding such as depth map construction.

Each camera is modeled as a pinhole specified by a set of intrinsic parameters describing the projection of the local coordinate frame onto the focal plane.  The relative orientation between coordinate systems is a rigid body transformation comprising the extrinsic parameters.

Two methods of stereo camera calibration are presented.  The traditional method establishes a world coordinate system and computes the mapping functions relative to the calibration rig.  From these individual mappings a relative transformation is extracted for use in rectification.  The conventional method often fails to reliably establish keypoint correspondences in highly asymmetric camera systems.  Therefore an additional method is described which directly computes the relative transformation without the use of a common coordinate frame.  The direct viewpoint matching method possesses the added benefit of single image rectification which preserves main camera image quality and reduces latency.

### Stereo configuration

Ideally, the optical axes should be parallel and the optical centers coplanar in a plane parallel to the focal plane.  The purpose of calibration is to correct any small deviations from this idealization.

Figure 10 - Asymmetric Stereo Configuration

Depending on the relationship between cameras, relative to the main camera the disparity vector points away from the auxiliary camera.

Figure 11 - Disparity Vector Polarity

### Normalized focal length

Let  and be the lens focal length at infinity, sensor horizontal resolution, sensor vertical resolution, and pixel size for the kth camera, respectively.  The dimensionless quantity known as the normalized focal length, e.g. in pixels, at infinity is

The normalized focal length determines the scale factor for the projective mapping of locations in space onto the sensor plane and thus is of fundamental importance in both calibration and rectification.

From this we compute the horizontal and vertical angles of views  and,

For cameras equipped with an autofocus mechanism any deviation from hyperfocus results in an optical scale change which must be accounted for by suitable modification of the normalized focal length.

The focused image appearing at distance  of an object at distance from the aperture will be displaced from the sensor plane by an amount  which is governed by the Gaussian lens equation

For the image distance is .  The displacement of the focusing point, e.g. the image location, from the focal plane is

,

hence the object appears blurred in the image formed on the sensor plane.  Next, consider the situation illustrated in Figure 12 in which the autofocus mechanism has translated the lens a distance away from the sensor plane in order to properly focus the object onto the sensor plane.

Figure 12 - Sensor Plane AF Shift

Because the object remains fixed the primed object distance after lens movement is

The lens displacement is selected such that the new image is in focus at the fixed sensor plane which located a distance f away from the original lens position. Measured from this new lens position the focused image is located at which must be

.

Inserting  and  into  and solving for the displacement yields the correct value of the lens shift

Figure 13 - Typical Lens Barrel Shift Values

Equivalently, the given a known lens shift, the object distance is about equal to

The optical magnifications for the blurred focal plane image, the AF image, and the focused original image, respectively, are obtained using the similar triangles in Figure 12 as follows:

The differential change in magnification of the image, e.g. at the focal plane, due to the lens shift is

Expanding the denominator of the second term we may rewrite  as the product of two terms

The first term represents the magnification due to the lens whereas the second term accounts for the physical effect of the slight change in object distance.  For mobile cameras the maximum lens shift is on the order of 100 um and the closest object distance is around 10 cm, hence the second term results in a maximum change of one part in a thousand.  Given that the second term is close to unity over the operating range coupled with the fact that the object distance is seldom known in practice we neglect it in the calculation of  and write the differential magnification as

It is common usage to refer to the new image distance  as the modified focal length and denote it as.

In view of the above analysis the normalized focal length at calibration for the kth camera is

,

where is the modified focal length at capture.

Typically, calibration is performed at a lower resolution.  Therefore the most general expression for  requires an additional digital scale factor  which accounts for the increased region of space subtended by the new larger virtual pixels.  Combining both effects results in a total scale factor embodied by the normalized focal length at calibration,

The expression in  allows compensation for arbitrary lens position and processing resolution during calibration.  Additional scaling factors applied during rectification may be accommodated by suitable modification of  which is discussed in Section 21.12.1.

## Camera Calibration

Traditional calibration, also known as full calibration, involves calculation of transformations relative to a fixed world coordinate system embedded in a calibration rig.  The calibration rig enables robust keypoint detection in 3D space and ensures that calibration parameters are observable from an estimation standpoint.  Four coordinate systems are employed for expressing the world points as they progress through the camera system: 1) The common frame (world),2) the local camera frame, 3) local sensor frame (in the focal plane), and 4) the digital image buffer (pixel).

The method presented here is based on the special structure of the pinhole model whose solution is in terms of a homogenous linear system.  The following equations are for theoretical illustration and due to the high sensitivity of the estimation problem to noise do not represent a robust solution.  For example estimation of the sensor principal point is notoriously sensitive to errors in keypoint position.  In later sections a nonlinear iterative method is presented which minimizes the geometrically intuitive notion of reprojection error.

The method employs the direct linear transformation (DLT) which simultaneously solves for both camera pose (extrinisics) and intrinisics (focal length, scale, image center) based on known 3D world point.  The resulting equations define a homogenous linear equation with solution via least squares and SVD.

A point located with respect to a world coordinate system with origin  has coordinates  in the local camera frame related by

In   is the matrix which aligns the local to world axes and  is the displacement of the local optical center from .  When P is the local origin, P0 must be C.The matrix

is defined by three Euler angles: pitch , yaw , and roll about the x,y and z axis, respectively.  The angles are measured according to the right hand rule and appear as counter clockwise rotations when the axis about which they occur point toward the observer.  The transformation  may be represented in coordinate form as

Or in matrix form using the projection function  and homogenous coordinates as

where the rigid body transformation is

Note the values for p are in units of say mm.  Measurements for calibration are in the form of pixel coordinates of keypoints located in the image.  A transformation between focal plane and image coordinates is required as illustrated in Figure 14 - Pixel coordinates.

Figure 14 - Pixel coordinates

The focal plane coordinate  in mm with  , is related to the image coordinate in pixels with by

where account for the potential misalignment of the principal point of the lens module with the center of the image sensor.  Combining  with  yields the overall mapping from world to pixels

Note that u,and v in pixels are defined as pixels with origin centered on the image sensor plane, e.g. the actual buffer coordinates (I,j) are related to (u,v) by

When computing the estimation matrix from known pixel locations (I,j) centering first reduces estimation error, though in principle it is possible to work in non centered pixel coordinates in which case the actual center (u0,v0) will be estimate instead of the offset .

Defining the intrinsic camera parameter matrix as

the normalized focal lengths

and invoking the shorthand in   we write the final non-linear transformation between image and world coordinate as

where

is the linear portion of the mapping containing both intrinsic and extrinsic parameters. In component form  is

which we form into a linear homogenous system for estimation of the calibration parameters.

Given N keypoints   corresponding to world points  the 2N homogenous equations

are formed into a matrix equation in 12 unknown parameters

of which only 7 are independent (3 angles, 3 translations, and normalized focal length). The 2N x 12 linear system is

where the data matrix given by

Note the Z coordinates of the world points cannot be zero or the matrix will be not be full rank.  Thus a 3 dimensional chart is needed for single snapshot calibration.  The homogenous least squares solution to  is well known to be given by the eigenvector  of   corresponding to the smallest eigenvalue, ie,

The solution can be found by computing the SVD of H

in which case

Given  is homogeneous we have solved only up to a scale factor. Identifying the rows ,  and matching through  yields the partition

Matching the partitions in  we arrive at

The distance tz should be positive, hence the sign of rho is

Using the fact that R is unitary, we deduce

Similarly,

Given r3 direction is fixed from b3, we use cross product relationships for perpendicular vectors rk

Crossing  the rows of  with the third and using

Taking magnitudes,

and back into substituting into yields

When either r1 or r2 is found the other can be computed from .

Solving , the translation becomes

The estimated extrinsic matrix is

And intrinsic is

The baseline is estimated from

Where L is the chart feature size.

The sign   of rho is determined from knowledge of the fact that the origin of the world coordinate system is in front of the camera

Table 2

| Parameter | Value |
| --- | --- |
| B |  |

Method of Zhang

Note that in  since R is orthogonal by definition

Rho^2B*B’ = K*K’ can be soloved for elements of K

Note, finding rks by cross product yields significant error.  Better set center offset to zero and to find by rk’s by dot product

### Stereo Camera Calibration

For stereo cameras two sets of intrinsic and extrinsic parameters are computed.  The extrinsic parameters align the viewpoint of each camera with the world coordinate axes.  Let the rotation matrix, intrinsic matrix, and camera centers for the kth camera be , respectively.  Then the mapping for each 3D point is

Denote the calibration rig keypoints as  .  We then solve  for each camera and follow the procedure for extracting the parameters given in table ().

Based on the calibration parameters we can project image 2 onto the pose of image 1.  From Appendix A the matrix which orients camera 2 such that it matches the viewpoint of camera 1 is

Figure 15 – Dual Camera Rectification

Including translation the complete mapping between points is

Where the relative translation is

### Relative Viewpoint Matching

In viewpoint matching we do not seek to obtain the individual transformations between the cameras and the world coordinates.  Instead we move directly to compute the projective transform between camera 1 and 2 by minimizing the vertical disparity.  This method is equivalent to constraining epipolar lines to have equal vertical coordinate.  This method is ideal for highly asymmetric stereo cameras in which differing lens and camera characteristics can negatively impact correspondence matching.

Figure 16 - Single Camera Correction

In conventional stereo calibration two individual camera matrices are computed with respect to the rig coordinate system which are then applied to each camera separately during rectification.  In our analysis this method has two drawbacks, in particular for asymmetric systems.  Firstly, applying the transformation to the main image reduces image quality.  Secondly, due to differing lens and sensor characteristics the resulting estimated transformation matrices can be inaccurate.

In our method, a single transformation is computed with respect to the main camera.  This is the “relative viewpoint matrix” R indicated in the figure below.  In this method the transformation from aux to main is directly computed and applied only to the aux image.  As a result the estimation of the matrix is more accurate and main image quality is preserved.  In this case the translation vectors of the optical centers and the baseline are not computed.  Current assembly tolerances for optical center placement are sufficiently small such that the error induced by not accounting for this baseline difference relative to the specification is smaller than the error caused by unknown lens tilt and hysteresis within the autofocus mechanism.  The compelling measure of system performance is not contained in the number of parameters estimated (e.g. optical centers, principal points, etc.) but in the resulting correspondence between posterior transformation depth and disparity.  Our tests have shown that after rectification our method results in better distance/disparity relationships and hence more accurate depth maps.

### Minimal Intrinsic parameters

In viewpoint matching we do not seek to obtain the individual transformations between the cameras and the world coordinates.  Instead we move directly to compute the projective transform between camera 1 and 2.  To that end, note that the vertical coordinate of the point in camera 2’s focal plane maps to

in camera 1’s focal plane.

Given a collection of keypoints in the focal plane of camera 2 (aux)  we are able to devise a projective transformation to points

in focal plane 1 which minimizes the mean squared vertical disparity

where

defines the projection of the vertical coordinate.  Given K points multiplying  and collecting terms yields the linear homogenous system

The solution to  is again the eigenvector  of  corresponding to the smallest eigenvalue which is computed via the SVD of H.  With,   for scale homogenous scale factor and sign .  Since rotation matrices have positive diagonal elements, or equivalently an negative element implies as a angle shift by 180

where we have used the orthogonality constraint of  and the vector cross product ensures  a right handed coordinate system.  The view alignment matrix is

### Focal Length Estimation

In cases where f2 is known more accurately than f1, for example if f1 corresponds to autofocus camera whereas f2 is fixed, we may estimate f1 by incorporating it into the parameter vector.  For numerical stability we work with the ratio  and formulate a modified version of

In this case we have

### Vertical offset

If we choose to estimate a principal point offset

Defining

We reuse the machinery developed in ().

And solve the Kx6 linear system

Where the data matrix is

And  .  Adopting the notation in ()

From  matching matrices we have

Crossing  the first row of rho*B with the third and using

Taking magnitudes,

and back into substituting into yields

### Yaw Correction

Since this method is only weakly sensitive to yaw angles, we provide an additional estimate of the yaw (rotation in the optical plane) angle of camera 2 (auxilliary).  The difference between the center disparity and the mean disparity observed about a vertical line passing through the world origin is used to compute the yaw angle.  Both the horizontal and vertical field of views must be known.  Let D be the distance in mm between the  keypoints  located on the vertical line passing through the world origin on the chart.  Given the square size  the vertical length in mm of the line is

The corresponding length of the line in pixels at the focal plane is

From Figure 17 the distance in mm subtending the vertical field of view  is

and the distance to the chart is

Figure 17 - Center Line Geometry

The horizontal distance in mm of the line subtending the horizontal field of view of the reference camera,   is

Let

Be the disparity at the ijth pixel.  The disparity at the center of the chart assuming no yawing is

Figure 18 - Geometry for yaw calculation

From Figure 18 the yaw tilted angle is

Where

Is the residual disparity at the center due to unaccounted for yaw,

Is the measured average center disparity, Z is the depth at the center of the chart, W is the horizontal field of view in mm, and w is the sensor width in pixels.  The updated relative orientation matrix is

### Horizontal Offset

Calculation of horizontal principal point relative offset.  Any deviation from zero in the principal point will appear as a fixed offset which will be manifested as an additional tilt angle

corresponding to this offset.  Because the offset causes shifts in the same direction as the tilt we cannot recover it from the data.  Using the structure of the chart and known distance of the origin we estimate the keypoint distances and disparities

The observed disparities are,

From which we estimate the axis shift

### Geometric Correction

### Dual Chart

Traditionally, geometric correction is included in the camera intrinsic parameters via a low order polynomial for both radial and tangential distortion.  During rectification the geometric correction is applied to both images.  The quality of the geometric undistortion depends on the degree to which the actual lens distortion matches the polynomial model.

Alternatively, the viewpoint aligning projective transformation may be applied to a flat checkerboard chart.  Coplanar points should have equal disparity, therefore after subtracting a planar fit any remaining disparity is interpreted as geometric distortion. The residual disparity is fit through the keypoints by a suitable surface.  Low order polynomials estimated via regularized regression and thin plate splines yield good results.

3) The geometric surface parameters account for the relative geometric distortion between aux and main lenses.  This fit is represented by n coefficients describing a polynomial surface.  This surface is applied on the aux image during rectification and therefore preserves main image quality compared with individually applying lens correction.  The standard method of lens correction calculates radial and tangential distortion coefficients and therefore assumes the observed geometric distortion conforms to this model.  Our method is not limited by a model and therefore can support arbitrary lens profiles.

As an example, consider the estimation of a 3rd order polynomial via least squares.  The maximum degree for all polynomial termsis such that which results in a 10 term polynomial

From M keypoints pairs we express the disparities at location  in the aux image in terms of the surface as

In matrix form  is represented by the Mx10 system

Where the kth row of H is

The least squares solution for the coefficients in  is

where  is the pseudo inverse of .  For a point   in the image the interpolated disparity is

and the updated position is

For numerical stability we work in normalized coordinates

’

which can be written in terms of a normalization matrix  N via .  Additionally, the geometric correction may be computed in both x and y yielding a 2D correction vector

### Single Chart

We assume the handset is placed relative to the chart such that only yaw is present.  This assumption is met for example when mechanical elements are used to rigidly place a module during factory calibration.  Knowledge of the chart structure may be used to estimate true depth if a reference depth is also known.  The distance  to the chart center perpendicular to the baseline, e.g. post rectification, is used as the reference distance.  The estimated disparity for the kth keypoint is  from which we compute the estimated depth  .

Measuring in this frame to points on the left side of the chart the kth keypoint will have actual depth

Solving, using least squares for the cosine of the angle

The correct distance is therefore  .  The corresponding correct disparity is

The geometric error manifold is then

as before.

Similarly, for the right chart,

### Rectification

### Normalized focal length computation

Images may be rectified at arbitrary resolution and focal length, however, the resulting scale changes must be accounted for.  The captured image for the kth camera with native sensor resolution  is scaled by the image processor toand then center cropped to the output resolution.   The lens position is adjusted by the autofocus mechanism by distance  as described in Section 21.7 such that the capture modified focal length becomes

The normalized focal length  with respect to the resolution and focal length  at calibration is modified to

which accounts for both the optical and digital scale changes.  Note that only the scaled resolution , not the output cropped resolution ,  affects the normalized focal length computation due to the fact that only modification of the pixel size influences the projective.

If the normalized focal length adjustment reported by the AF is given in terms of the ratio  then  becomes

,

where  is the ratio at calibration time.

### Dual Camera Correction

In single camera correction the rectification transform is applied only to a single camera, for example if camera 1 is chosen as the reference image then camera 2 is transformed.  While this has advantages in terms of simplicity and preserves the original fidelity of one image, in most situations the method is very slightly suboptimum.  Aside a slight reduction in the joint field of view, given that depth measurements based on triangulation are computed with respect to the baseline perpendicular connecting C1 to C2, this method will result in small errors depending on the deviation of the reference camera optical axis from the baseline perpendicular.  If we define the baseline coordinate system, e.g. the module, with respect to the line connecting the camera optical centers, then all alignment errors will appear with respect to this frame.

Figure 19 - Single Camera Correction Geometry

The steering of camera 2 toward 1 based on a single rotation causes an apparent shift of camera 2’s optical center.  For a yaw only correction depicted in Figure 19 the shift  is in the XZ plane with magnitude

Full stereo rectification in the sense of horizontal epipolar lines involves dual camera correction.  Consider the kth camera (local) coordinate Pk is related to its world coordinates P by

Here  is always defined in the world coordinate system and is therefore the matrix which rotates the world axes   to align with the camera axis , e.g.  .  If we define

then the estimator provides us with the matrices

Note, according to  the estimator natively provides  (not ).

Substituting, the relationship for P2 into P1 in , we find the relationship between points detected in frame 2 in frame 1 coordinates are

If we define the relative orientation matrix which converts points in RF2 to RF1 as

,

and the relative translation vector which is specified in terms of RF1 (because R has already been applied) as

,

then the relative rigid body transformation is

The transformation from RF2 to RF1 coordinates is described by simple matrix multiplication

Note that  defines the baseline direction whose length is.

Next, we introduce another reference frame with respect to the module.  In this frame the cameras are rigidly affixed and are mutually affected by any transformation applied to the module.  Let M1 and M2 defined above and M0 be the camera 1, camera 2, and module rigid body transforms respectively.  As always they are linked by the world coordinate system embedded in the calibration rig.  Let R10 and R20 be the rotations of the cameras with respect to the module.  Let nz1 and nz2 be the optical axes directions for the cameras, and nz0 the normal to the baseline  .  With these definitions in place we may write

Figure 20 - Rectification Coordinate Systems

Thus the relative rotation matrix is

Using  the matrix which aligns the axes of RF1 should equal the axes of RF1

Using  in  we find that

Using  and  in

Therefore the rectification matrix which aligns to the module is simply  which is applied to both cameras.  In terms of axes, using  the module axes is related to the camera axis by the similarity transformation

Figure 20 describes the stated relationships pictorially.

Construction of module rectification matrix uses the baseline direction vector T as a reference to define two additional perpendicular directions.  The end result of the application of R10 ensures that the baseline with respect to RF1 has only a horizontal component, e,.g. in

We construct the matrix as follows.  Define

which is by definition such that , and

### Single Camera Correction

### Full Calibration

From full calibration we obtain the rectification rigid body transformation

as well as the camera matrices . The goal is to use auxiliary depth information derived either from passive triangulation or active methods to overlay one image on top of another.  Depth information is used to find the camera coordinates in the reference frame for which the depth is valid.  This 3D point is then reprojected on the focal plane of camera 2.  For the camera matrices to simplify rectification we can write the optical centers as the sum of the nominal center based on image resolution and the offset as

Given depth information Z1 in camera frame 1 corresponding to sensor coordinates x1,y1,f1, we first compute the camera coordinates

Using the relative rigid body matrix M we find the equivalent point in camera 2 frame

Hence we find the digital buffer coordinates

Technically, the buffer coordinates also have a rounding function which maps to nearest pixel.  During rectification interpolation is used to blur this error.

In matrix shorthand if we use  then

And

Combining  and

The rendered image in the viewpoint of camera due is constructed with a suitable interpolation scheme over all pixels in the image.

Given calibration rigid body transformations

It is easily shown that the (backward) rectification matrix

### Relative Viewpoint Method

During rectification we apply the relative transformation to the auxiliary image to render its viewpoint into the reference camera viewpoint.

The actual application of the correction of camera 2 to the view of camera 1 proceeds as

where  .  To work in homogeneous coordinates with  we may factor out the focal lengths via the matrix

where  denotes elementwise multiplication.  In S, f is the projection or destination focal length and f’ is the source or current focal length.  Using the projective mapping function defined in  we write the output pixel coordinates as

When applying the transformation during rectification we use the inverse mapping.  With the understanding that

and

we can perform he rectification via bilinear interpolation mapping pixels in the destination image to their corresponding values in the source image.

In terms of standard calibration notation the forward transformation is

Including centers

### Rectification with Geometric Correction

The above rectification procedure may be used to accommodate the geometric correction described in .  The total mapping  becomes

,

where the geometric term is given by the fitting function

,

which is described in Section 21.11.

### Rectification with Arbitrary Module Orientation

In situations where the capture viewpoint of the stereo camera module deviates from the calibration orientation by the permutation matrix  the equations for rectification are modified by the similarity transformation between M and P.  The revised equations become

For example, for a 90 clockwise rotation the P matrix is

### Calibration Using a Non-Planar Chart

A non-planar chart contains two checkerboard planes connected at right angles.  Such a chart is contains sufficient geometric structure such that a single capture may be used for estimating calibration parameters.  The following example uses the parameters listed in ().

Figure 21 - L-chart geometry

### Mechanical Tolerances

The camera module must be encased in an immobilization bulwark of sufficient rigidity such that any force resultant deformation does not cause appreciable depth error.  A force applied at one end will cause a torque which slightly rotates the camera thus inducing an inaccurate disparity reading.

A deflection angle of .02895 degrees, or equivalently a linear vertical deflection of 0.0253 mm will cause 1 cm of error at 1 meter for a baseline of 50 mm.

B = 50;

Z0 = 1000;

dZ = 10;

d = .02895*pi/180

f = 4;

delta_alpha = atand( (Z0+dZ)/B ) - atand(Z0/B)

B*delta_alpha*pi/180

Cp = [B*cos(d),-B*sin(d)];

cp = [B*cos(d)+f*sin(d),-B*sin(d)+f*cos(d)];

cp0 = [B,f];

Cp0 = [B 0];

mcp = -tan(d);

mCp = (Cp(2)-Z0)/(Cp(1)-0);

%intersection of rotated system

X1 = (cp(2)-Z0-mcp*cp(1))/(mCp-mcp)

Z1 = mCp*X1+Z0

%X1= (f*cos(d)-B*sin(d)-Z0-cot(d)*(B*cos(d)+f*sin(d)))/( (Z0+B*sin(d))/(B*cos(d)) - cot(d) );

%Z1 = Z0+ ( Z0+B*sin(d) )/(B*cos(d))*X1;

D = sqrt( (X1-cp(1))^2 + (Z1-cp(2))^2 );

m3 = -f/D;

Xp = 0; %@TX

Zp = m3*(Xp-Cp0(1)) + Cp0(2);

deflection = Cp(2)

dZ = Z0-Zp

### Error Analysis

### Yaw

Yaw rotates an optical axis perpendicular to the vertical axis.  For small yaw  applied to Camera 2, R2 takes the form

Consider a plane at distance Z0=1000mm from Camera 1’s optical center at the origin with identical Camera 2 located at (-B,0,0).  The equation of the plane is Z=Z0, hence a point P on the plane wrt to Camera 1 has coordinates P = (X Y Z0), and the sensor coordinates

If we compute the distance assuming parallel optical axes the result is

At X=0, the depth error is

For a horizontal cut at Y=0, the reconstructed plane  exhibits hyperbolic curvature

If the distance can be estimated independently, for example using a known object and the FOV, it is possible to extract the yaw angle either by measuring the erroneous depth profile or using normal information.

close all;clear;

phi = 60;

Lx = 1000;

Ly = Lx;

dx = 10;

Z_chart = 1000;

B = 50;

n = [0 0 -1]';

[xx,yy] = meshgrid(-Lx/2:dx:Lx/2,-Ly/2:dx:Ly/2);

zz = Z_chart*ones(size(xx));

[N,M] = size(xx);

figure(1)

h_plane = plot3(xx(:),zz(:),yy(:),'.');hold on;axis equal;

n0 = [0;0;Z_chart];

quiver3(n0(1),n0(3),n0(2),n(1),n(3),n(2),100,'g');

set(gca,'zdir','reverse')

xlabel('X');ylabel('Z');zlabel('Y');

P_chart = [xx(:) yy(:) zz(:)]';

T_chart = [0;0;Z_chart];

%C2..0..C1....->+X

C0 = [0 0 0]';

dC2 = [0 0 0]';

C2 = [-B 0 0]' + dC2;

C1 = C0;

phi1 = 0;

phi2 =0;

psi1 = 0;

psi2 = -.5;

theta1 = 0;

theta2 = 0;

omega1 = [psi1 phi1 theta1]';

omega2 = [psi2 phi2 theta2]';

R1 = get_matrix(pi/180*omega1(2),pi/180*omega1(1),pi/180*omega1(3));

R2 = get_matrix(pi/180*omega2(2),pi/180*omega2(1),pi/180*omega2(3));

n1 = R1*n;

p = 2e-3;

h = 1520;

w = 2688;

H = p*h;

W = p*w;

f = 4.47;

fbar = f/p;

c1 = [w/2;h/2];

c2 = [w/2;h/2];

K10 = [f 0 0; 0 f 0;0 0 1];

K1 = [fbar 0 0;0 fbar 0;0 0 1];

K2= K1;

K1inv = inv(K1);

K2inv = inv(K2);

P = P_chart;

for i=1:length(P)

P0 = P(:,i);

P1 = R1'*(P0-C1);

P2 = R2'*(P0-C2);

P20 = eye(3)'*(P0-C2);

p1 = ProjectPoint(K1*P1);

p2 = ProjectPoint(K2*P2);

q1 = ProjectPoint(K10*P1);

q2 = ProjectPoint(K10*P2);

d(i) = p2(1)-p1(1);

Zhat = fbar*B./d(i);

y = pi/180*psi2;

Zhat_theory = -(B)/((P1(1))/P1(3) - ((B + P1(1) - P1(3)*y))/(P1(3) + y*(B + P1(1))));

Zhat_theory2 = -B/(P1(1)/P1(3)-(B+P1(1)-P1(3)*y)/P1(3)*(1-y*(B+P1(1))/P1(3)));

%Zhat_theory3 = -B/(y - B/P1(3) - (B*y^2)/P1(3) + (B^2*y)/P1(3)^2 - (P1(1)*y^2)/P1(3) + (P1(1)^2*y)/P1(3)^2 + (2*B*P1(1)*y)/P1(3)^2);

Zhat_theory3 = -B/(y - B/P1(3) + (P1(1)^2*y)/P1(3)^2 + (2*B*P1(1)*y)/P1(3)^2);

Zhat_theory4 = P1(3)/(1-P1(3)*y/B);

P1out(:,i) = Zhat*K1inv*p1;

P1out_theory(:,i) = Zhat_theory*K1inv*p1;

P1out_theory2(:,i) = Zhat_theory2*K1inv*p1;

P1out_theory3(:,i) = Zhat_theory3*K1inv*p1;

P1out_theory4(:,i) = Zhat_theory4*K1inv*p1;

P1true(:,i) = IntersectRays([q1(1:2);f],[q2(1:2);f],R1,R2,C1,C2);

P1est(:,i) = IntersectRays([q1(1:2);f],[q2(1:2);f],eye(3),eye(3),C1,C2);

eZ(i) = P1true(3,i)-P1out(3,i);

eZ_theory(i) = -P1true(3,i)^2*y/B/(1-P1true(3,i)*y/B);% - Zhat_theory;

end

P1out_array = reshape(P1out',N,M,3);

P1out_theory_array = reshape(P1out_theory',N,M,3);

P1out_theory2_array = reshape(P1out_theory2',N,M,3);

P1out_theory3_array = reshape(P1out_theory3',N,M,3);

P1out_theory4_array = reshape(P1out_theory4',N,M,3);

a = 1+P(1,:).^2./P(3,:).^2+2*B*P(1,:)./P(3,:).^2;

b = B./P(3,:);

psi2hat = (b - B./P1out(3,:))./a;

mean(psi2hat)

figure(2),plot(P1out_array((N-1)/2,:,1),P1out_array((N-1)/2,:,3));hold on;

plot(P1out_theory_array((N-1)/2,:,1),P1out_theory_array((N-1)/2,:,3));hold on;

plot(P1out_theory2_array((N-1)/2,:,1),P1out_theory2_array((N-1)/2,:,3));hold on;

plot(P1out_theory3_array((N-1)/2,:,1),P1out_theory3_array((N-1)/2,:,3));hold on;

plot(P1out_theory4_array((N-1)/2,:,1),P1out_theory4_array((N-1)/2,:,3));hold on;

figure,plot(P1true(1,:),eZ);hold on;plot(P1true(1,:),eZ_theory);

figure,imagesc(reshape(P1out(3,:),size(zz)));

%plot reconstructed pointcloud

figure,

plot3(P1out(1,:),P1out(3,:),P1out(2,:),'.');axis equal;hold on;

quiver3(P1out(1,(N*M-1)/2),P1out(3,(N*M-1)/2),P1out(2,(N*M-1)/2),n1(1,:),n1(3,:),n1(2,:),100);

set(gca,'zdir','reverse')

xlabel('X');ylabel('Z');zlabel('Y');

### Realtime Calibration

Realtime, self, or online calibration is the process of calibrating a system without a specialized calibration rig.

### Rotation Matrices

Rotation matrices are defined with respect to a right handed coordinate system.  The Euler matrices which rotate a vector about one of the three canonical axes according to the right hand rule (fingers curling around thumb pointed in positive direction of axis gives positive rotation) are

which define rotation about the X,Y,Z axes, respectively.

Example: Rotation about Y-axis

Rotation about the Y-axis rotates vectors in a clockwise fashion according to the right hand rule.  If the direction of significance is along the Z-axis such a rotation is known as yaw.

Since directions perpendicular to the rotation axis are unaffected, the Y-axis is unchanged and the columns of the result form the linear transformation with respect to the original basis.

To find the coordinates of a point in one coordinate system with respect to another first translate the source point by the shift in coordinate frame centers (the center is written source variables as addition between quantities expressed in different coordinate systems is meaningless), ie .  Next find the distances along each of the destination axes by projecting the point onto the destination axes written in terms of the source coordinate system, ie. .  In the destination system the point is

To check this consider the point  under  , then solving for P0 gives (0,0,Z) as expected.  Next, with C=0, consider in destination coordinates which should yield 2X rotation in P0 coordinates.  Indeed

### Coordinate Transformation

Orthonormal basis vectors u, and v, e.g. coordinate axes, with respective coordinates eta,xi

Since x is the same point in space we may arrive at its location with respect to any basis.  Equating () and (), and applying orthonormality we find the coordinates are related by

where

is the angle between vectors vi and uj.  If we collect the coordinates into an ordered list then we can write  as

where the subscript indicates which basis the coordinates are with respect to.

Example: 2D rotation.

Let  be the standard basis and  be the counter clockwise rotation of u1 and u2 in the xz plane.  Then from

as illustrated in ().

If a third basis is added , with coordinates  then converting from 2 to 3 can be done through 1 via

If 1 is the standard basis, then

### Applying Image Transformations

There are two parts to applying a spatial transform: 1) specifying the new pixel location by a spatial mapping function, and 2) determining the corresponding intensity value at the mapped location.  Task one is typically some type of geometric transformation such as translation, rotation, or projection.  Task two is accomplished by a resampling operation such as nearest neighbor or bilinear interpolation.

The forward spatial mapping directly relates corresponding pixels   in a source or original image S1 to generally non-integer coordinates  in a destination or deformed image S2 by a function

Using the nearest neighbor intensity value in the source image the complete mapping takes the form

This approach suffers from the creation of undesirable gaps or holes, e.g. no information in the source for a given output location, and overlaps, e.g. non-unique mappings between input and output pixels in some regions.  ,We find the coordinate in image 1 which maps to coordinate in source image 2 which we are generating such that

If  , then , where  is the matrix supplied for the transformation routine which will accomplish .

### 2-D Calibration

Stereo cameras in two dimensions provide a conceptual understanding of calibration which is intuitive and easy to visualize.   Consider two pinhole cameras with focal lengths and optical centers  defined wrt to a world coordinate system.  The baseline separation is

The world coordinates representing a 2-D point  are  and the local coordinates of the kth camera are .  The kth camera is rotated counter clockwise wrt C0 by  .  From section () the transformation matrices are

Which is the standard ccw rotation matrix.

Coordinates of the point P0 in C0 frame in Ck frame are therefore

The points Pk are projected on the focal planes at  via the pinhole projection equation

To form the points on the sensor frame .  The world coorindates are

The corrected coordinates, e.g. coordinates of p2 wrt C1 are found by first converting p2 to C0, and then to C1, via the transformation

The reprojected coordinates of p2 wrt C1 on the plane at  is

Where  and  is

In world coordinates this point is

We may also find the point  which is the point along the ray intersecting the focal plane f1 in the C2 and the point along the ray intersecting f2 where

In accordance with pinhole geometry the magnification corresponding to the intersection of the ray with the focal planes in C2 at f1 and f2 is

And the distance between is

### Least Squares Solution of Homogeneous Equations

The least squares solution to the overdetermined linear system

minimizes the distance from the column space of A to b, e.g. .  The so called homogenous system has b=0 in which case the minimizing solution is trivial.  Therefore a constraint must be applied in order to yield a useful solution.  A convenient constraint is to require x to have unit norm.  In this case we solve the constrained minimization problem

Diagonalizing the symmetric matrix A’A using its eigendecomposition ,

Where the rotation  does not change the magnitude of x,  .  Therefore we find  is bounded by

where  and are the min and max eigenvalues, respectively.  The minimum occurs for  or equivalently  .  In other words the solution is the eigenvector corresponding to the smallest eigenvector of (p.d.) A’A which is geometrically is the minor axis of the quadratic form   which traces out a hyperellipse.

### L-Chart Simulation Code

clear;close all;

%Y down, Z toward camera, X right

L = 50.8;

N = 15;

B = 30;

Z1 = 1000;

Z2 = Z1;

f1 = 5;

f2 = 2.5;

D1 = 4000/1280;

D2 = 1920/1280;

a1 = 1.12e-3*D1;

b1 = a1;

a2 = 2e-3*D2;

b2 = a2;

R0 =get_matrix(0,-pi/4,0);%right hand rule about y axis

dC1 = [0 0 0]';

dC2 = [0;0;0];

C0 = R0*[0;0;Z1];

C1 = R0*[B/2 0 Z1]' + dC1;

C2 = R0*[-B/2 0 Z2]'  + dC2;

h1 = 3000/D1;

w1 = 4000/D1;

h2 = 1080/D2;

w2 = 1920/D2;

Vfov1 = 2*atan(h1*a1/(2*f1));

Vfov2 = 2*atan(h2*a2/(2*f2));

Hfov1 = 2*atan(w1*a1/(2*f1));

Hfov2 = 2*atan(w2*a2/(2*f2));

dc1 = 0;

dc2 = dc1;

p10 = dc1*[a1;b1]; %center offset in mm

p20 = dc2*[a2;b2];

c10 = p10./[a1;b1];

c20 = p20./[a2;b2];

c1 = 0.5*[w1 h1]' + c10;

c2 = 0.5*[w2 h2]' + c20;

nz = [0 0 1]';nx = [1 0 0]';ny = [0 1 0]';

%

theta1 = 4/2*pi/180*[1;1;1];

theta2 = -4/2*pi/180*[1;1;1];

R10 = get_matrix(theta1(1),theta1(2),theta1(3));

R20 = get_matrix(theta2(1),theta2(2),theta2(3));

R1 =  R0*get_matrix(0,0,0)*R10;

R2 =  R0*get_matrix(0,0,0)*R20;

T1 = -R1'*C1;

T2 = -R2'*C2;

M1 = [R1 C1;0 0 0 1];

M2 = [R2 C2;0 0 0 1];

R = R1'*R2;

T = R1'*(C2-C1);

nz1 = R1*nz;nx1 = R1*nx;ny1 = R1*ny;

nz2 = R2*nz;nx2 = R2*nx;ny2 = R2*ny;

nz0 = R0*nz;

kappa1 = 0*1e-2;

kappa2 = kappa1;

v = 0;%kp noise in pixels bernoulli v*[-1,0,1]

[XXleft,YYleft,ZZleft] = meshgrid(-N:0,-(N-1)/2:(N-1)/2,0);

[XXright,YYright,ZZright] = meshgrid(0,-(N-1)/2:(N-1)/2,1:N);

PPleft = [XXleft(:)';YYleft(:)';ZZleft(:)'];

PPright = [XXright(:)';YYright(:)';ZZright(:)'];

PP = [PPleft PPright];

Ns = size(PP,2);

PP = L*PP;

figure,plot3(PP(1,:),PP(2,:),PP(3,:),'b.');hold on;axis equal;grid on;axis ij;

xlabel('x');ylabel('y');zlabel('z');

quiver3(0, 0, 0,L*nz(1), L*nz(2), L*nz(3),'m','MaxHeadSize',10);

quiver3(0, 0, 0,L*nx(1), L*nx(2), L*nx(3),'k','MaxHeadSize',10);

quiver3(0, 0, 0,L*ny(1), L*ny(2), L*ny(3),'k','MaxHeadSize',10);

quiver3(C1(1), C1(2), C1(3),L*nz1(1),L*nz1(2), L*nz1(3),'m','MaxHeadSize',10);

quiver3(C1(1), C1(2), C1(3),L*nx1(1), L*nx1(2), L*nx1(3),'k','MaxHeadSize',10);

quiver3(C1(1), C1(2), C1(3),L*ny1(1), L*ny1(2), L*ny1(3),'k','MaxHeadSize',10);

quiver3(C2(1), C2(2), C2(3),L*nz2(1),L*nz2(2), L*nz2(3),'m','MaxHeadSize',10);

quiver3(C2(1), C2(2), C2(3),L*nx2(1), L*nx2(2), L*nx2(3),'k','MaxHeadSize',10);

quiver3(C2(1), C2(2), C2(3),L*ny2(1), L*ny2(2), L*ny2(3),'k','MaxHeadSize',10);

plot3(0,0,0,'o','Color',[1 .5 0])

plot3(C1(1),C1(2),C1(3),'.g','MarkerSize',10);

plot3(C2(1),C2(2),C2(3),'.r','MarkerSize',10);

view(0,90);camorbit(20,30,'data',[0 1 0])

q1 = v*(round(3*rand(2,Ns)-3/2));

q2 = v*(round(3*rand(2,Ns)-3/2));

Tfov = 0*1/4;

S1 = zeros(h1,w1);

S2 = zeros(h2,w2);

k1 = 0;k2 = 0;

for i=1:Ns

Pi = PP(:,i);

P1 = R1'*(Pi-C1);

P2 = R2'*(Pi-C2);

x1 = f1*P1(1)/P1(3);

y1 = f1*P1(2)/P1(3);

x2 = f2*P2(1)/P2(3);

y2 = f2*P2(2)/P2(3);

r1 = sqrt(x1^2+y1^2);

r2 = sqrt(x2^2+y2^2);

x1d = x1/(1 + kappa1*r1^2);

y1d = y1/(1 + kappa1*r1^2);

x2d = x2/(1 + kappa2*r2^2);

y2d = y2/(1 + kappa2*r2^2);

u1 = x1d/a1 + c1(1) + q1(1);

v1 = y1d/b1 + c1(2) + q1(2);

u2 = x2d/a2 + c2(1) + q2(1);

v2 = y2d/b2 + c2(2) + q2(2);

i1 = round(v1);

i2 = round(v2);

j1 = round(u1);

j2 = round(u2);

if i1>1 && j1>1 && i1<h1 && j1<w1

k1 = k1 + 1;

S1(i1,j1) = 1;

PP1(:,k1) = Pi;

p1(:,k1) = [v1;u1;P1];

end

if i2>1 && j2>1 && i2<h2 && j2<w2

k2 = k2 + 1;

S2(i2,j2) = 1;

PP2(:,k2) = Pi;

p2(:,k2) = [v2;u2;P2];

end

end

Ifov1 = find(p1(1,:)>Tfov*h1 & p1(1,:)<(1-Tfov)*h1 & p1(2,:)>Tfov*w1 & p1(2,:)<(1-Tfov)*w1);

Ifov2 = find(p2(1,:)>Tfov*h2 & p2(1,:)<(1-Tfov)*h2 & p2(2,:)>Tfov*w2 & p2(2,:)<(1-Tfov)*w2);

xyz1 = [p1(1:2,Ifov1);PP1(:,Ifov1)/L];

xyz2 = [p2(1:2,Ifov2);PP2(:,Ifov2)/L];

figure,

subplot(1,2,1); imagesc(filter2(ones(5)/5^2,S1)); title('Camera 1 Keypoints');axis equal;

axis([1 w1 1 h1]);

subplot(1,2,2); imagesc(filter2(ones(5)/5^2,S2)); title('Camera 2 Keypoints');axis equal;

axis([1 w2 1 h2]);

[u1hat,v1hat,A1,H1] = SolveHomogeneousSystem(xyz1,h1,w1);

[u2hat,v2hat,A2,H2] = SolveHomogeneousSystem(xyz2,h2,w2);

[R1invhat,K1,T1hat] = ExtractParametersFull(A1);

[R2invhat,K2,T2hat] = ExtractParametersFull(A2);

K1 = [1 0 w1/2;0 1 h1/2;0 0 1]*K1;  %re-center points

K2 = [1 0 w2/2;0 1 h2/2;0 0 1]*K2;  %re-center points

T1hat = -L*T1hat; %minus sign because camera frame is pointing away from orign

T2hat = -L*T2hat;

R1hat = inv(R1invhat);

R2hat = inv(R2invhat);

Rhat = R1invhat*R2hat;

M1invhat = [R1invhat T1hat; 0 0 0 1];%=[R1' R1'*C1]

M2invhat = [R2invhat T2hat; 0 0 0 1];

M1hat = inv(M1invhat);

M2hat = inv(M2invhat);

That = T1hat-Rhat*T2hat;

Mhat = M1invhat*M2hat;

Minvhat = inv(Mhat);

P1hat = ApplyRigidBodyTransform(M1invhat,PP);

figure;%world coordinate points

plot3(P1hat(1,:),P1hat(2,:),P1hat(3,:),'.');

hold on;axis equal;grid on;axis ij;

plot3(p1(3,:),p1(4,:),p1(5,:),'.','Marker','x');

legend('estimated','truth');

title('Camera 1 Reconstruction: World->Local');

xlabel('x');ylabel('y');zlabel('z');

P1hat = ApplyRigidBodyTransform(M1hat,p1(3:5,:));

figure;%world coordinate points

plot3(P1hat(1,:),P1hat(2,:),P1hat(3,:),'.');

hold on;axis equal;grid on;axis ij;

plot3(PP(1,:),PP(2,:),PP(3,:),'.','Marker','x');

legend('estimated','truth');

title('Camera 1 Reconstruction: Local->World');

xlabel('x');ylabel('y');zlabel('z');

%analysis stereo rectification

R1hat*R2invhat*[nx2 ny2 nz2]-[nx1 ny1 nz1]%=0

[xyz1,xyz2] = MatchKPLabels(xyz1,xyz2,5);

n = size(xyz1,2);

xyz1 = [0 1 0 0 0;1 0 0 0 0;0 0 1 0 0;0 0 0 1 0; 0 0 0 0 1]*xyz1;%swap x and y

xyz2 = [0 1 0 0 0;1 0 0 0 0;0 0 1 0 0;0 0 0 1 0; 0 0 0 0 1]*xyz2;

p21hat = ProjectCamera(K1*Rhat/K2*[xyz2(1:2,:);ones(1,n)]);%p2 truth projected into p1 RF

p21hat_sc = ProjectCamera(K1*eye(3)/K2*[xyz2(1:2,:);ones(1,n)]);%p2 scaled only into p1 RF

dhat = p21hat-xyz1(1:2,:);

xyz0 = [zeros(2,Ns);PP/L];

xyz21 = [p21hat;xyz1(3:5,:)];

xyz21_sc = [p21hat_sc;xyz1(3:5,:)];

[xyz21,xyz0] =MatchKPLabels(xyz21,xyz0,5);

I0 = find(xyz21(3,:)==0 & xyz21(4,:)==0 & xyz21(5,:)==0);

Zhat = K1(1,1)*B./dhat(1,:);

P21hat = ApplyRigidBodyTransform(Minvhat,p1(3:5,:));

figure;%world coordinate points

plot3(P21hat(1,:),P21hat(2,:),P21hat(3,:),'.');

hold on;axis equal;grid on;axis ij;

plot3(p2(3,:),p2(4,:),p2(5,:),'.','Marker','x');

legend('estimated','truth');

xlabel('x');ylabel('y');zlabel('z');

title('Camera 1 -> Camera 2');

view(0,90);camorbit(25,45,'data',[0 1 0])

P12hat = ApplyRigidBodyTransform(Mhat,p2(3:5,:));

figure;%world coordinate points

plot3(p1(3,:),p1(4,:),p1(5,:),'.','Marker','x');

hold on;axis equal;grid on;axis ij;

plot3(P12hat(1,:),P12hat(2,:),P12hat(3,:),'.');

legend('estimated (2-','truth');

title('Camera 2 -> Camera 1 World points');

xlabel('x');ylabel('y');zlabel('z');

view(0,90);camorbit(35,45,'data',[0 1 0])

figure,plot(xyz21_sc(1,:),xyz21_sc(2,:),'.');axis ij;hold on;grid on;

plot(xyz21(1,:),xyz21(2,:),'.r');

plot(xyz21_sc(1,I0),xyz21_sc(2,I0),'.g');

plot(xyz21(1,I0),xyz21(2,I0),'.k');

legend('Camera 2 KP','

Ztruth = DistanceToBaselinePlane(L*xyz0(3:5,:),nz1,C1);

eZ = Ztruth - Zhat;

eZ12_save = eZ(I0);

figure,plot(Ztruth);hold on;plot(Zhat);legend('truth','estimate');

figure,plot(eZ)

figure,plot(xyz21_sc(2,:)-xyz1(2,:),'.-');hold on;

plot(dhat(2,:),'.-');

title('Vertical Disparity');

legend('Before','After');

%% rectify both optic axes perp to baseline vector wrt module perpendicular

e1 = -That/norm(That);

e1 = e1/norm(e1);

e2 = [-e1(2) e1(1) 0]';

e2 = e2/norm(e2);

e3 = cross(e1,e2);

R120 = [e1';e2';e3'];

%

% p20hat = ProjectCamera(K1*(R10*R1'*R2)/K2*[xyz2(1:2,:);ones(1,n)]);%p2 truth projected into p1 RF

% p10hat = ProjectCamera(K1*R10/K1*[xyz1(1:2,:);ones(1,n)]);%p2 truth projected into p1 RF

p20hat = ProjectCamera(K1*(R120*Rhat)/K2*[xyz2(1:2,:);ones(1,n)]);%p2 truth projected into p1 RF

p10hat = ProjectCamera(K1*R120/K1*[xyz1(1:2,:);ones(1,n)]);%p2 truth projected into p1 RF

xyz10 = [p10hat;xyz1(3:5,:)];

xyz20 = [p20hat;xyz2(3:5,:)];

[xyz10,xyz20] =MatchKPLabels(xyz10,xyz20,5);

I0 = find(xyz20(3,:)==0 & xyz20(4,:)==0 & xyz20(5,:)==0);

dhat0 = xyz20(1:2,:)-xyz10(1:2,:);

Zhat0 = K1(1,1)*B./dhat0(1,:);

Ztruth = DistanceToBaselinePlane(L*xyz10(3:5,:),nz0,C1);

eZ = Ztruth - Zhat0;

eZ120_save = eZ(I0);

figure,plot(Ztruth);hold on;plot(Zhat0);legend('truth','estimate');

figure,plot(eZ)

figure,plot(p10hat(1,:),p10hat(2,:),'.');axis ij;hold on;

plot(p20hat(1,:),p20hat(2,:),'.r');

plot(p20hat(1,I0),p20hat(2,I0),'.g');

plot(p10hat(1,I0),p10hat(2,I0),'.k');

Consider N lines in 3D

The distance from a point  to the line is

The point which minimizes the sum of squared distances from the point to the lines is found by minimizing the objective function

Eqn () can be rexpressed in terms of matrices as

where

and is a matrix of zeros. The choice of x determines the direction along which the distance points ti are referenced to.

In applications beyond simple geometric triangulation there will be additive uncertainty in the values of any of the quantities due to noise or unavoidable calibration inaccuracies.  Because of noise we employ the least squares solution to ()

where is related to the distance along the ith line where the perpendicular is drawn at  to the minimizing point . The distance between lines is .   The angle between the ith line and the vector from the pi(tihat) and optimum point given by

should be 90 degrees.  For two lines there are 6 equations in 5 unknowns.  As N increases the dimension of the column space of H grows as 3N while the parameter dimension grows as N.  Therefore as N grows the nullspace of the data matrix becomes asymptotically empty, and the estimate improves.( explain better…

Example:

Angle between p1(t1hat) and r and p2(t2hat) (black line) is 90 degrees as it should be.

T1 vs t2 quadratic surface distance between the two lines.

The answer simplified using Mathematica is

b(f2(2u1(u1-u2)+(v1-v2)2)+2u1v2(-u2v1+u1v2))2(f2((u1-u2)2+(v1-v2)2)+(u2v1-u1v2)2),b(f2(u1-u2)(v1+v2)+2v1v2(-u2v1+u1v2))2(f2((u1-u2)2+(v1-v2)2)+(u2v1-u1v2)2),bf(2f2(u1-u2)-(v1+v2)(u2v1-u1v2))2(f2((u1-u2)2+(v1-v2)2)+(u2v1-u1v2)2),b(f2(-u1+u2)+v2(u2v1-u1v2))f2((u1-u2)2+(v1-v2)2)+(u2v1-u1v2)2,b(f2(-u1+u2)+v1(u2v1-u1v2))f2((u1-u2)2+(v1-v2)2)+(u2v1-u1v2)2

This expression is much more complicated than using the standard assumption of perfectly aligned cameras.  When the y values for each camera are equal,as in the standard case, the result is equal to the standard geometricly derived result.

{bu1u1-u2,bv1u1-u2,bfu1-u2,b-u1+u2,b-u1+u2}

All coordinates in the retinal frame are given by

Where i is the column index, j is the row index, alpha is the scale that maps pixels to length, f is the focal length, M is the horiztonal number of pixels, and N is the vertical pixel dimension, and is the normalized inverse focal length.  In this way we can represent points in the local retinal frame as 2D vectors or 3D vectors always with a 1 as their 3rd coordinate.  Thus in eqn () the focal length factor cancels out and we can replace f in the equations with the normalized focal length.  This reduces the number of unknown parameters in the calibration step, assuming the vertical and horizontal scales are equal.  Therefore in the equations for x,y,z the u’s and v’s can be replaced by actual image coordinates i ,j translated by (N+1)/2,(M+1)/2.

f = 30;

b = 31;

N = 972;

M = 1024;

i1 = N/2+5;

j1 = M/2+1;

i2 = N/2 - 5

j2 = M/2 - 1;

u1 = 30;

v1 = 30;

u2 = 20;

v2 = 20;

p10 = [0;0;0];

p20 = [b;0;0];

p1 = [u1;v1;f];

p2 = [u2;v2;f];

t1 = 0:.01:1;

t2 = 0:.01:2;

k1 = p1-p10;

k2 = p2-p20;

figure(1);

for i=1:length(t1);

l1(:,i) = p10 + t1(i)*(p1-p10);end

for i=1:length(t2)

l2(:,i) = p20 + t2(i)*(p2-p20);

end

for i=1:length(t1)

for j=1:length(t2)

d(i,j) = norm(l1(:,i) - l2(:,j))^2;

end

end

plot3(l2(1,:),l2(2,:),l2(3,:),'r');grid on;hold on;

plot3(l1(1,:),l1(2,:),l1(3,:),'b');

xlabel('X');ylabel('Y');zlabel('Z');

H = [eye(3) k1 zeros(3,1);eye(3) zeros(3,1) k2];

x = [p1;p2];

thetahat = inv(H'*H)*H'*x

rhat = thetahat(1:3);

t1hat = thetahat(4);

t2hat = thetahat(5);

[d00 t00] = min(d(:));

[it10 it20] = ind2sub(size(d),t00);

t10 = t1(it10)

t20 = t2(it20)

p1hat = p10 + (1-t1hat)*k1;

p2hat = p20 + (1-t2hat)*k2;

p1hat0 = p10 + t10*k1;

p2hat0 = p20 + t20*k2;

plot3(thetahat(1),thetahat(2),thetahat(3),'gd');

plot3(p1hat(1),p1hat(2),p1hat(3),'mo');

plot3(p2hat(1),p2hat(2),p2hat(3),'mo');

line([thetahat(1) p1hat(1)],[thetahat(2) p1hat(2)],...

[thetahat(3) p1hat(3)],'Color','m');

line([thetahat(1) p2hat(1)],[thetahat(2) p2hat(2)],...

[thetahat(3) p2hat(3)],'Color','m');

line([p1hat0(1) p2hat0(1)],[p1hat0(2) p2hat0(2)],...

[p1hat0(3) p2hat0(3)],'Color','c');

line([p1hat(1) p2hat(1)],[p1hat(2) p2hat(2)],...

[p1hat(3) p2hat(3)],'Color','k');

plot3(p1(1),p1(2),p1(3),'om');plot3(p2(1),p2(2),p2(3),'om')

plot3(p10(1),p10(2),p10(3),'om');plot3(p20(1),p20(2),p20(3),'om')

a1 = acosd( (p1hat-p2hat)'*k1/norm(k1)/norm(p1hat-p2hat))

a2 = acosd( (p2hat-p1hat)'*k2/norm(k2)/norm(p2hat-p1hat))

a10 = acosd( (p1hat-rhat)'*k1/norm(k1)/norm(p1hat-rhat))

a20 = acosd( (p2hat-rhat)'*k2/norm(k2)/norm(p2hat-rhat))

figure(2)

mesh(d);

figure(3)

imagesc(d);hold on;

plot(it20,it10,'kx')

## Structure From Motion

In stereo calibration, known 3D points are used to compute camera pose and and intrinsics.  SFM is the reverse procedure which attempts to estimate the world position of objects from calibrated cameras.  The general problem is projected points provide no depth information.  Triangulation can be used, but this requires camera pose and translation/baseline knowledge.  We will investigate three methods: homogenous and non-homogenous least squares, reprojection error.  One might be inclined to try intersecting lines (as in section opti theory), however, this leads to a complicated non-linear function due to the projection.

A single calibrated camera observes the projection of a point P across N views with known rotation + translation.  In this case, the goal is to estimate the world position from the feature locations pn = (xn,yn) on the image plane.  The point (together with the camera intrinisics and the N-rigid body transformations Mn which tell how the camera orientation has changed realtive to the world) and its N projections define N lines in space.  The intersection is the value of P.  In the general more useful case where the rigid body transformations between views are unknown, triangulation is not possible and the problem is hard.  Also, the same point must be identified and matched in each view and pixel errors will invariably prevent a perfect intersection and cause mismatches.  We'll get to that, but first let's go through this at least historically interesting solution as an exercise.

The feature point  in the nth camera frame is related to its world frame representation  by

where  and X is the homogenous representation of r.  (note rn is location of camera center relative to world in world, and pn is same relative to camera).  Next, the relationship between image point and the expected location of world point on the image plane requires conversion via the intrinsic matrix K followed by projection which we combine into the 3x4 camera matrix which operates on homogenous vector X.

The pixel positions are then

#### Reprojection error

The most accurate

where yhat is the model and y are the observed values

Can be minized by linearizing followed by iteration.

### Direct Linear Transformation (DLT)

Homogenous least squares

Solve via total least squares (section ()).  Stack up to form H

Least squares

In this case we fix W to be equal to one.

Let ai^T be the rows of the upper left 3x3 matrix residing in P, then by isolating r, from () we form the linear system , with

The solution is .

Because the error is Z times reprojection error also numerical issues with magnitudes of the elements of the linear system.  Can be perverse geometries.

Line of 3 cameras separated by 1 m.  Object at 2 m.

### Shape from X

Involves the relationship between 3D spatial information and its 2D projection on the image plane.  If pk is the 2d point in the kth view of 3d point P are related by the pinhole camera equation pk = h(P).  points pk and pj are related by a rigid body transformation M = R,T.  If M is known P can be triangulated.  Stereo cameras with known extrinsics and intrinsics are a special case of this.  Conversely, if P is known, M can be found.

the relationship P = RP + T can be differentiated to yield

Pdot = RdotP + RPdot + Tdot

The  linear velocity is related to the angular velocity of a point in the camera frame located by T wrt a reference frame is

Pdot = omega x P + T.

Suppose we have N views of a point P with projections pk. For the case of K views, with camera poses M

### Iterative Closest Point

Iterative closest point (ICP) computes the pairwise alignment in the form of a rigid body transform between two point clouds.  The steps are select points from the overlap region, match points to establish correspondence, reject and/or weight point pairs, construct error function, minimize error function.

Consider a set of point clouds  where  denotes the kth member of the nth pointcloud.  The identical world point  is related to its corresponding point  in the mth pointcloud by rotation matrix R and translation vector t

In general, point correspondences are unknown between different point clouds.  Therefore, ICP consists of two parts: data association, e.g. matching, and parameter estimation.  The matching step attempts to establish correspondence between two identical points in space measured in different coordinate frames.  The estimation portion finds the optimum rigid body relationship between the two clusters according to an appropriate error metric.

Typically nearest neighbor matching is employed.  For matching function u the corresponding points are

### Point to Point Distance Error Metric

For two point clouds let  and be a set of of corresponding points.  Then we seek to estimate R and t which will align, e.g. bring to the same reference coordinate system, by minimizing the error function

Differentiating wrt to t we find that for any R the estimate for t is

Where

Substituting  into  yields the orthogonal Procrustes problem (OPP) discussed in ()

Where

Is the mean centered point cloud.

Letting  we may recast  in terms of the Frobenius norm as

The solution to the OPP is via the SVD of

with R selected as

Using the matrix factorization in () we can write the minimum error as

Larger singular values decrease the estimation error.

### Point to plane distance Error Metric

In the point to plane metric the distance from the source point x to the corresponding destination point x’ is measured along the normal vector of the plane tangent at the destination point.

The metric is

To defeat the nonlinear nature of the expression we approximate R in its infinitesimal form by expanding about .  Thus the expressions for small rotations about x,y,z, respectively are

Neglecting higher order terms, for any ordering, the aggregate differential rotation becomes

where the skew symmetric matrix which generates the cross product for with any vector  is defined as

Using the linear expression  the rotation operation on any source point x becomes

and substituting into  yields

Where

For Nx6 matrix and Nx1 the solution is via least squares

function [R, T] = MinimizePointToPlaneMetric(p, q, nv)

% Set up the linear system

d = -(p - q);   %3 x N, columns are ith 3 x 1 difference

b = dot(nv,d,2);  %b(i) = nv(:,i)'*d(:,i)

A = [cross(nv,p) nv]; %N x 6

%least squares solutiono

X = (A'*A)\A'*b; %x=[thetax,thetay,thetaz,tx,ty,tz]

cx = cos(X(1));

cy = cos(X(2));

cz = cos(X(3));

sx = sin(X(1));

sy = sin(X(2));

sz = sin(X(3));

T = X(4:6);

R = [cy*cz, sx*sy*cz-cx*sz, cx*sy*cz+sx*sz; %ensure R is unitary

cy*sz, cx*cz+sx*sy*sz, cx*sy*sz-sx*cz;

-sy,          sx*cy,          cx*cy];

Some insight into the spatial configurations which effect performance is gleaned by expanding the cost function about its minimum whose shape is controlled by the covariance matrix  ,

Thus for small excursions  the change in E will be .  Good performance in terms of robustness, stability, and minimum error are obtained for pointcloud formations which cause large changes in E for small changes in the alignment parameters.  The eigenvalues of A’A can be used to measure such properties.  Large eigenvalues in all directions leads to a steep cost function and hence high robustness, and vice versa for small or combinations of large and small eigenvalues.

### Global Alignment

Depth map has noise in Z, pointcloud has noise 1/Z.

## Appendix A: Code

clear;

J = 3;

B = 1000;

fl = 10;

K = [fl 0 0;0 fl 0;0 0 1];

rn = [B*(0:J-1);0*rand(1,J)*B/2;0*rand(1,J)*B/6];

T = 1000;

phi = 0;

theta = pi/10;

psi = 0;

for i=1:J

R = utils.e2R(phi,theta,psi); %r = R*p + t (R: C to W)

RR(:,:,i) = R;

P(:,:,i) = K*[R.' -R.'*rn(:,i)];

end

r = [B*(J-1)/2+50;0;2*B];

sigma = 0:0.5:2;

for n=1:length(sigma)

for t=1:T

for i=1:J

p(:,i) = utils.TF(r,RR(:,:,i),rn(:,i));

y(:,i) = (utils.PIz(p(:,i),fl)) + sigma(n)*randn(2,1);

end

rhat = PnP(P,y);

rhat2 = utils.TriangulateLS(y,P);

e1(t) = 1/sqrt(3)*norm(rhat - r);

e2(t) = 1/sqrt(3)*norm(rhat2 - r);

end

E1(n) = mean(e1);

E2(n) = mean(e2);

end

figure,plot(sigma,E1,'LineWidth',2);hold on;grid

plot(sigma,E2,'LineWidth',2);

ylabel('RMSE/B');xlabel('Pixel STD');

title('DLT Comparison')

legend('Homogenous LS','Standard LS')

function rhat = PnP(P,y)

H = [];

N = size(y,2);

for n=1:N

xn = y(1,n);

yn = y(2,n);

Pn = P(:,:,n);

a1 = Pn(1,:);

a2 = Pn(2,:);

a3 = Pn(3,:);

Hn = [a3*xn - a1; a3*yn-a2];

H = [H; Hn];

end

[U,S,V] = svd(H);

rhat = V(:,end);

s = rhat(4);

rhat = rhat/s;

rhat = rhat(1:3);

end

classdef utils

methods(Static)

function Omegav = Omega(v)

Omegav = [0 -v(3) v(2);v(3) 0 -v(1);-v(2) v(1) 0 ];

end

function Gammaq = Gamma(q)

q0 = q(1);

v = q(2:4);

Gammaq = q0*eye(4) + [0 -v.';v -utils.Omega(v)];

end

function Rq = R(q)

Rq = (2*q(1)^2-1)*I + 2*(q(2:4)*q(2:4).') + 2*q(1)*Omega(q(2:4));

end

function r = FF(p,R,t)

n = size(p,2);

r = R*p + t*ones(1,n);

end

function p = TF(r,R,t)

n = size(r,2);

p = R.'*(r-t*ones(1,n));

end

function y = PIy(p,f)

y(1,1) = f*p(1)/p(2);

y(2,1) = f*p(3)/p(2);

end

function y = PIz(p,f)

y(1,1) = f*p(1)/p(3);

y(2,1) = f*p(2)/p(3);

end

function H = Hc(p,f)

z = p(3);x=p(1);y=p(2);

H = zeros(2,3);

H(1,1) = f/z;

H(2,2) = f/z;

H(1,3) = f*x/z^2;

H(2,3) = f*y/z^2;

end

function I = CheckFOV(p,r,u,phi)

I = 0;

d = u.'*(p-r);

theta = acos(d/norm(p-r));

if abs(theta)<phi/2 && d>0

I = 1;

end

end

function Dx = D(r,q)

Dx =  [q(1)*r(1) + q(3)*r(3) - q(4)*r(2) q(2)*r(1) + q(3)*r(2) + q(4)*r(3) q(1)*r(3) + q(2)*r(2) - q(3)*r(1) q(2)*r(3) - q(1)*r(2) - q(4)*r(1);

q(1)*r(2) - q(2)*r(3) + q(4)*r(1) q(3)*r(1) - q(2)*r(2) - q(1)*r(3) q(2)*r(1) + q(3)*r(2) + q(4)*r(3), q(1)*r(1) + q(3)*r(3) - q(4)*r(2);

q(1)*r(3) + q(2)*r(2) - q(3)*r(1) q(1)*r(2) - q(2)*r(3) + q(4)*r(1), q(4)*r(2) - q(3)*r(3) - q(1)*r(1) q(2)*r(1) + q(3)*r(2) + q(4)*r(3)];

end

function q = qwT(w,T)

if sum(w)==0

q = [1;0;0;0];

else

s = norm(w);

u = w/s;

q = [cos(s*T/2); u*sin(s*T/2)];

end

end

function pq = qMult(p,q)

u = p(2:4);

v = q(2:4);

p0 = p(1);

q0 = q(1);

pq = [p0*q0-u.'*v;p0*u + q0*v + cross(u,v)];

end

function q = R2q(R)

T = 1 + trace(R);

r11 = R(1,1);r12=R(1,2);r13 = R(1,3);

r21 = R(2,1);r22=R(2,2);r23 = R(2,3);

r31 = R(3,1);r32=R(3,2);r33 = R(3,3);

if (T >10^8)

S = 2*sqrt(T);

a= S/4;

b= (r32 -r23)/S;

c= (r13- r31)/S;

d= (r21- r12)/S;

else

if (r11 >r22) && (r11 >r33)

S = 2*sqrt(1 + r11- r22- r33);

a= (r23- r32)/S;

b=-S/4;

c= (r21- r12)/S;

d= (r13- r31)/S;

elseif (r22 >r33)

S = 2*sqrt(1- r11 + r22- r33);

a= (r31- r13)/S;

b= (r21- r12)/S;

c=-S/4;

d= (r32- r23)/S;

else

S = 2*sqrt(1- r11- r22 + r33);

a= (r12- r21)/S;

b= (r13- r31)/S;

c= (r32- r23)/S;

d=-S/4;

end

end

q = [a;b;c;d];

end

function Rq = q2R(q)

Rq = (2*q(1)^2-1)*eye(3) + 2*(q(2:4)*q(2:4).') + 2*q(1)*utils.Omega(q(2:4));

end

function R = e2R(phi,theta,psi)

c2 = cos(theta);

s2 = sin(theta);

R = [c2 0 -s2;0 1 0;s2 0 c2];

end

function r = TriangulateLS(p,P)

K= size(p,2);

H = [];d=[];

for k=1:K

if sum(abs(p(:,k)))>0

uk = p(1,k);

vk = p(2,k);

Pk = P(:,:,k);

a1 = Pk(1,1:3);

a2 = Pk(2,1:3);

a3 = Pk(3,1:3);

Hk = [a3*uk-a1; a3*vk-a2];

dk = [Pk(1,4)-uk*Pk(3,4); Pk(2,4)-vk*Pk(3,4)];

H = [H; Hk];

d = [d; dk];

end

end

r = inv(H.'*H)*H.'*d;

end

function r = TriangulateLSy(p,P)

K= size(p,2);

H = [];d=[];

for k=1:K

if sum(abs(p(:,k)))>0

uk = p(1,k);

vk = p(2,k);

Pk = P(:,:,k);

a1 = Pk(1,1:3);

a2 = Pk(2,1:3);

a3 = Pk(3,1:3);

Hk = [a2*uk-a1; a2*vk-a3];

dk = [Pk(1,4)-uk*Pk(2,4); Pk(3,4)-vk*Pk(2,4)];

H = [H; Hk];

d = [d; dk];

end

end

r = inv(H.'*H)*H.'*d;

end

end

end